Starlink or other satellite ISPs make sense when you need connectivity where wired/cellular options are weak or as an independent backup that stays online when local infrastructure fails, but they are usually inferior to good fiber/cable for latency consistency, reliability, and long-term cost.​

## When satellite is worth it

- Rural / remote locations with no good wired options: Starlink can deliver tens to hundreds of Mbps with ~20–40 ms latency in places that only have DSL, WISP, or nothing at all.​

- Infrastructure-independent backup: Because the path is dish → space → ground station, it does not depend on local fiber, poles, or last-mile cabinets, making it valuable as a failover when backhoes, regional fiber cuts, or power issues take terrestrial ISPs down.​

- Disaster resilience and field work: Businesses and agencies use Starlink as part of disaster-recovery and remote-operations kits (construction sites, mines, emergency response, etc.) where temporary, quickly-deployable links are needed.​

## Benefits of Starlink specifically

- Much lower latency than traditional GEO satellite: ~20–40 ms vs ~600+ ms for classic satellite, because Starlink runs in low Earth orbit.​

- Higher throughput than legacy satellite: average user speeds have climbed into the 100+ Mbps range in many regions, significantly above older VSAT offerings.​

- Mobility and flexible deployment: Kits can be moved, mounted on vehicles, or used at seasonal/temporary sites, unlike fixed wired connections.​

## Shortcomings and risks of Starlink

- Weather sensitivity (rain fade): Using Ku/Ka bands means heavy rain or dense storm clouds can cause slowdowns, packet loss, or brief outages; this is less severe than traditional GEO but still noticeable for critical real-time use.​

- Less consistent than good fiber/cable: Even in clear weather, latency jitter and occasional brief dropouts are more common than on quality terrestrial fiber, which can be a problem for competitive gaming or very latency-sensitive workloads.​

- Data management and congestion: Starlink has implemented various traffic-management and soft-cap policies; at busy times, users can see throttling or slower speeds versus advertised peak performance.​

## Using Starlink as primary vs backup

| Use case | Starlink as primary | Starlink as backup |
| --- | --- | --- |
| Rural / no wired option | Often the best available choice. ​ | Less relevant; usually becomes de-facto primary. ​ |
| Urban with fiber/cable | Usually worse than fiber for latency/jitter. ​ | Strong independent failover path. ​ |
| Mission-critical uptime | Risk from weather and brief dropouts. ​ | Excellent alongside fiber/5G in multi-WAN setup. ​ |
| Mobile / temporary site | Very attractive, easy to move. ​ | N/A (it is primary by design in the field). ​ |

## Practical guidance

- Use Starlink as primary only if your alternatives are poor: e.g., long-loop DSL, oversold WISP, or nothing at all.​

- Use Starlink as backup if you already have good fiber/cable: pair it with a multi-WAN router (or 5G + Starlink) so failover and load-balancing are automatic; this gives you true path diversity that a second circuit on the same poles cannot.